# Essential Peripherals — INT-1540

Week 10 · Module 10: Essential Peripherals

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Compare monitor resolutions, panel types, and refresh rates

• Identify USB standards and connector types by speed and shape

• Distinguish display interface cables and their capabilities

• Describe KVM switches and their lab use cases

## Key Terms

IPS

In-Plane Switching — wide viewing angles and accurate color reproduction.

USB Type-C

24-pin reversible connector — speed depends on host port spec, not connector shape.

KVM switch

Keyboard-Video-Mouse switch — shares one set of peripherals among multiple computers.

HDMI 2.0

18 Gbps AV interface — supports 4K at 60 Hz maximum.

DisplayPort 1.4

32.4 Gbps display interface — supports 4K@120Hz with DSC.

## The Concept

Peripherals are the user-facing layer of the PC. Monitor quality is defined by resolution (1080p/1440p/4K), panel type (IPS for color, TN for speed, VA for contrast), and refresh rate (60 Hz baseline; 144+ Hz for gaming). Display cable choices matter: VGA is legacy analog; DVI-D is digital-only; HDMI carries audio + video; DisplayPort is the high-refresh standard. HDMI 2.0 caps 4K at 60 Hz — you need HDMI 2.1 or DisplayPort 1.4 for 4K@120Hz+. USB-C is a connector shape, not a speed guarantee — verify the port spec before assuming it is USB 3.2 or Thunderbolt.

## Worked Examples

Example 1:

[object Object]

## Common Mistakes to Avoid

• Assuming all USB-C ports deliver USB 3.2 speed — a budget laptop's USB-C may be USB 2.0.

• Using HDMI 2.0 on a 4K 120Hz monitor — you will be limited to 60 Hz regardless.

• Connecting a second monitor to the motherboard when a discrete GPU is installed — use GPU outputs for both.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. Which panel type has the widest viewing angles and best color accuracy?

• A. IPS (In-Plane Switching)

• B. TN (Twisted Nematic)

• C. VA (Vertical Alignment)

• D. OLED (Organic LED)

2. HDMI 2.0 supports 4K at what maximum refresh rate?

• A. 60 Hz

• B. 120 Hz

• C. 144 Hz

• D. 240 Hz

3. USB Type-C is best described as:

• A. A reversible connector shape — speed depends on port spec

• B. A guaranteed USB 3.2 Gen 2 standard

• C. A connector exclusively for Thunderbolt 4

• D. A legacy connector being replaced by USB-A

4. A KVM switch serves what purpose in a multi-computer lab?

• A. Share keyboard, monitor, and mouse between computers

• B. Expand USB ports on a single computer

• C. Split one video signal to multiple monitors

• D. Copy HDMI from one GPU to several displays

5. A VGA connector carries which signal type?

• A. Analog video only

• B. Digital video and audio

• C. Digital video only

• D. Analog audio and video

## Self-Check Answers

1. A — IPS maintains consistent color output at angles up to ~178 degrees.

2. A — HDMI 2.0's 18 Gbps bandwidth supports 4K at 60 Hz maximum.

3. A — USB-C is a 24-pin oval connector shape — actual speed ranges from USB 2.0 to USB4 depending on the host port.

4. A — A KVM routes one set of keyboard/video/mouse peripherals to multiple computers via a toggle.

5. A — VGA transmits only an analog RGB signal — a legacy standard with no digital or audio capability.