# RAM — INT-1540

Week 4 · Module 4: RAM

Name (last name and class year only): ___________________________________

This companion enhances the PGCC curriculum; it does not replace it. Canvas is the official record of your grade.

Students with a 504 plan or IEP: your accommodations apply through PGCC Disability Support Services.

## Objectives

• Distinguish DDR4 and DDR5 RAM specifications

• Explain and configure dual-channel memory

• Identify DIMM vs SO-DIMM form factors

• Describe ECC RAM and its use cases

## Key Terms

Volatile

Loses all data when power is removed — the defining characteristic of RAM.

DDR4

288-pin desktop DRAM at 1.2V, 2133-3600+ MT/s.

DDR5

288-pin next-gen at 1.1V, 4800+ MT/s, on-die ECC.

Dual-channel

Two matched DIMMs in alternating slots — doubles effective bandwidth.

ECC

Error-Correcting Code RAM — detects and corrects single-bit errors; required in servers.

## The Concept

RAM is the system's working memory — data and instructions in active use live here. Unlike storage, it is volatile: power off means all data is lost. DDR4 and DDR5 are the two current desktop standards; DDR5 adds speed, lower voltage, and on-die ECC. Installing RAM in dual-channel configuration (matched pair in alternating slots) doubles the effective memory bus width, improving performance in memory-intensive tasks. ECC RAM adds extra bits to detect and correct single-bit errors — essential in servers, unnecessary overhead in consumer PCs. Always enable XMP/EXPO in BIOS to get the DIMM's advertised speed.

## Worked Examples

Example 1:

[object Object]

## Common Mistakes to Avoid

• Installing both DIMMs in adjacent slots 1+2 — creates single-channel mode.

• Not enabling XMP — RAM defaults to JEDEC 2133 MHz even if rated 3200+.

• Forcing a DDR5 stick into a DDR4 slot — different notch positions; forced insertion damages both.

## My Notes

Use this space to write notes during class. Focus on what your facilitator emphasizes.

## Self-Check Questions

Cover the answers below. Try each question before reading the answer.

1. 'Volatile' means RAM does what?

• A. Loses data when power is removed

• B. Preserves data without power

• C. Degrades speed over time

• D. Shrinks capacity as data fills it

2. Which RAM form factor is used in laptops?

• A. SO-DIMM

• B. Full 288-pin DIMM

• C. RIMM

• D. MicroDIMM

3. Dual-channel RAM configuration provides what benefit?

• A. Doubles effective memory bandwidth

• B. Increases capacity beyond module specs

• C. Reduces individual DIMM power draw

• D. Improves CPU cache performance automatically

4. ECC RAM is required in which environment?

• A. Servers requiring data integrity

• B. Gaming desktops needing max throughput

• C. Laptops needing compact size

• D. Consumer PCs with integrated graphics

5. To run DDR4-3200 at rated speed, which BIOS setting must be enabled?

• A. XMP (Extreme Memory Profile)

• B. Secure Boot

• C. Fast Boot

• D. AHCI mode

## Self-Check Answers

1. A — Volatile memory requires continuous power — RAM loses everything when the system powers off.

2. A — SO-DIMM (Small Outline DIMM) is the compact form factor for laptops and small-form-factor PCs.

3. A — Two DIMMs in matching channels activate a 128-bit effective bus — doubling bandwidth vs single-channel.

4. A — ECC is mandatory in servers and financial systems where single-bit memory errors could corrupt transactions.

5. A — XMP loads validated high-speed timing profiles stored on the DIMM into BIOS with one toggle.